How to update the MRX module firmware using a NOVA

To program the MRX-414 or MRX-214 plug in modules, place the MRX‐***.BIN file on an SD card, insert it into a transmitter and power it on. The transmitter can be an LA3 running version THD-218P or THD-304 or higher. The transmitter can also be a ZMT or Flex running ZMT-304 or higher.

Set the transmitter to “MONO” (not XR) reboot the transmitter if needed, turn off IFB TX mode and IFBRX‐to TX mode and the RX pulse timer so the IFB is OFF (or in RX mode). Make sure the MRX modules are all receiving the transmitter’s signal properly.

Be sure ENCRYPTION is turned off on the transmitter.

In the Nova RX menus set all the receivers that you want to program into SINGLE mode. Reboot the Nova and make sure the transmitter is being received reliably.

In the NOVA go to the RX SETUP :: QRX‐PROGRAM menu and observe the current running software in each module.

Press the * key to tell the receiver modules to start listening for a program. In the TRX or ZMT transmitter go to the menu labeled “PRESS UP TO SEND QRX PROGRAM”. Press UP in this menu to start sending the program. If the transmitter says No Program File Found the firmware may not be a sufficient version to allow the transmission of an MRX file. Make sure it is 218P or 304 or higher. If it refuses to send the program file the unit may be in XR mode instead of MONO mode or ENCRYPTION is turned on.

The Nova should say “FILLING” after about 15 seconds. Once the program is transmitted (10 minutes), the Nova will say “BURNING”. During this time DO NOT TURN OFF THE NOVA. Once the Nova says SUCCESS, wait at least 20 seconds and then power cycle the Nova. Check that the version is correct in the QRX PROGRAM menu.